SDxx - DUAL IN-LINE IC

SYMB	SMC	PINS	WID	PITCH	LEN	DESCRIPTION / ALIAS                                    _

SD1		4	300	200	---	VARO VM08 TYPE

SD2		4	300	300	---	CLOCK OSC TYPE

SD3		6	300	100	---

SD4		6	400	100	---	VDE SPACED OPTO

SD5	l	8	150	050	210

SD6	l	8	200	050	210

SD7	l	8	280	050	210

SD8		8	300	100	---

SD9		10	300	100	---

SD10	l	14	150	050	360	AD R-14

SD11		14	300	100	---

SD12	l	16	150	050	410	AD R-16A

SD13	l	16	300	050	410	ADW, AD R-16,AD811AR, EWE, LTC1181

SD14		16	300	100	---

SD15		16	300	100	---	DIP SWITCH SILK OUTLINE

SD16		18	300	100	---

SD17	l	20	280	050	510

SD18		20	300	100	---

SD19		22	300	100	---

SD20		22	400	100	---

SD21	l	24	150	050	610

SD22		24	300	100	---

SD23		24	600	100	---

SD24		28	600	100	---

SD25		40	600	100	---

SD26		64	900	075	---

SD27		8	300	100	---	8 PIN RELAY ON 14 PIN FOOTPRINT

SD28	l	24	280	050	610

SD29	l	48	300	025	625

SD30	l	16	150	025	215	QSOP

SD31	l	4	256	100	200	4 PIN OPTO, NEC PS2501L-1

SD32	l	20	150	025	335

SD33	l	16	280	050	410	NARROW ASW (SD13)

SD34	l	16	220	050	410	BI 628 SERIES RES NETWORK

SD35		20	300	100	---	DIP SWITCH SILK OUTLINE

SD36		40	---	---	---	40 LEAD TSOP ZIF SOCKET FOR SD59

SD37	l	8	140	050	210	ABSOLUTELY SMALLEST SD5

SD38	l	20	150	050	510

SD39		4	300	100	---	CRYSTAL CAN ON 14 PIN FOOTPRINT

SD40	l	24	300	050	610	SOW, SOG, MOT 751F SO28L

SD41	l	8	300	100	---	J-LEADED DIP

SD42	l	28	350	050	730	SOL

SD43	l	28	300	050	710	SOW,SOG

SD44	l	14	---	050	---	DG302 MIL FLATPACK

SD45	l	28	300	050	---	SOJ J-LEAD

SD46	l	20	300	050	510	SOL

SD47	l	8	---	050	---	BF08 MIL FLPK, DG201,UPG501B, FULL LD

SD48	l	6	300	100	---	J-LEADED DIP, LIKE SD41

SD49		40	300	100	---	NARROW DIP FOR PROM ADAPTER

SD50		28	300	100	---	Narrow DIP

SYMB	SMC	PINS	WID	PITCH	LEN	DESCRIPTION / ALIAS                                    _

SD51	l	6	150	050	160

SD52	l	20	170	25.6	260	NAT/SEMI TSSOP, SA625DK

SD53		6	300	100	---	SD39+2 GND PINS, SAWTEK 851xxx SER

SD54		6	200	100	---	RELAY ON 8 PIN FOOT-MOUSER 431-11xx

SD55	l	8	---	050	---	BF08 FLPK, DG201,UPG501B, SHORT LEAD

SD56	l	8	180	050	180	MACOM MAAM 37000-A12 CR3 PKG,BS=GND

SD57	l	8+	150	050	250	HMC158C8, HMC168C8

SD58	l	10	---	---	---	RFMT-56 MIXER

SD59	l	40	720	19.685	410	TSOP, ATMEL 27C1024, 40T JED MO-142-CD

SD60	l	40	480	19.685	410	VSOP, ATMEL 27C1024, 40V JED MO-142-CA

SD61	l	18	300	050	460	Toshiba SOP-18-P-375

SD62	l	15	800	050	700	L298 multi-watt sip, horizontal mount

SD63		6	300	100	500	PVI1050 opto coupler

SD64		6	300	100	650	Mouser 432-1605 pattern dip relay

SD65	l	32	430	050	830	Sram package

SD66	l	8	120	25.6	120	Generic MSOP (MM), Dallas uSOP, IR Micro-8

SD67		8	400	100	500	Wide 8 pin dip

SD68	l	14	300	050	400	Ceramic flat pack, ADQC-14, ADXL105JQC

SD69		16	300	100	1500	Potter-Brum T84 series 4PDT

SD70	l	10	120	19.7	120	AD uSOIC RM10, Maxim uMax

SD71	l	48	730	19.7	470	TSOP I 48-P-1220-0.50 (TC58128FT)

SD72	l	16	1000	100	1250	PLL Synth, EM Research BMS 190-0300-01

SD73	l	7	150	050	150	EMC Tech PST-01-A-1/PST-02-A-1, Castleated

SD74	l	16	750	080	750	PLL Synth, EM Research BMS 190-0300-01

SD75	l	4	350	560	560	Xinger hybrid, 11306-3, Castleated

SD76	l	6	220	100	310	Mini-Circuits ADP-2-20, Case CD542

SD77	l	16	180	25.6	210	Analog Devices RU-16 TSSOP-16

SD78	l					Some filter

SD79	l	16		100		Panasonic 4 channel optocoupler

SD80	l	6	180	050	180	Panasonic PS9701 optocoupler.

SD81	l	4	250	200	350	Diodes Inc DFxxxxSDI bridge rectifier

SD82	l	5	180	050	180	Panasonic PS9711 optocoupler.

SD83	l	8		100		Panasonic 2 channel optocoupler

SD84	l	28	210	25.6	410	Maxim 28 SSOP 21-0056

Notes:

1.	Width for through hole parts is row-row spacing, on smc parts it is body width

2.	Mixers Etc are in the MAG library

SAxx - AXIAL DIODES

SYMB	VERT	LEN	WID	PITCH	DESCRIPTION / ALIAS                                                _

SA1	l	---	100	100

SA2	l	---	100	150

SA3	l	---	100	200

SA4	l	---	200	250

SA5		150	075	250	1N4148 size diode

SA6		200	100	350	1N4000 series diode, 0.125 spacing minimum

SA7		250	100	400	1N4000 series diode, 0.125 spacing minimum

SA8		250	100	450

SA9		350	100	500

SA10		350	150	550

SA11		350	200	550

SA12		400	200	---

SA13		300	100	400

SA14		400	250	600	80SQ030, DO204AR, DO201AD

SA15		400	250	500		“		“

SA16	l	---	250	250		“		“

See SRxx for SMC axial diodes

SBxx - BRIDGE RECTIFIER

SYMB	SMC	PINS	CONFIG	DESCRIPTION / ALIAS                                                _

SB1

SB2

SB3

SB4

SB5

SB6	l	4	MOUSER RS6/GI KBU6 SMC MOD

SB7		4	DIGIKEY RS405LR 4A

SHxx - HEATSINK

SYMB	PACKAGE	DESCRIPTION / ALIAS                                                                         _

SH1	TO220		Square pattern 1 hole

SH2	TO3		Case shaped

SH3	TO220		Low profile

SH4	TO220		Vertical with 2 mounting posts

SIxx - INDICATOR/DISPLAY

SYMB	TYPE	H/V	DESCRIPTION / ALIAS                                                                         _

SI1	LED	V	Standard jumbo

SI2	LED	V	TIL307/308

SI3	VFD	V

SI4	VFD	V

SI5	LED	H	Jumbo / Long leads

SI6

SI7	LED	V	LED in square holder

SI8	LED	V	DIALITE 550-xx04 Series, same as SI7

SI9	LED	H	DIALITE 550-xx06 Series

SI10	VFD	V	6 Digit

SI11	LCD	H	BI 3-1/2 Digit DVM display, elastomer connect

SI12	LED	H	TIL 311

See SRxxx for SMC LED's

SOTxx - SO TRANSISTOR

SYMB		TYPE		PINS	DESCRIPTION / ALIAS 	                                   _

SO1		SOT23		3	ROHM SMD, SMT, SST

SO2		---		3	ROHM UMD, UMT, SC70-3

SO3

SO4		SOT89		3

SO5		SOT223	3+

SO6		DPAK		2+	TO252AA, SMC VERSION OF TO251AA

SO7		D²PAK/TO263	2+	CASE 418B-01, TO 263-3

SO8		SOT23		3	LORAL CASE 154 DIODE

SO9		TO220		2	2 LEAD VERTICAL MOUNT (PIN 1/3)

SO10		TO220		3+	D²PAK	STYLE FOOTPRINT/FULL TAB

SO11		TO220		2+	D²PAK	STYLE FOOTPRINT/FULL TAB

SO12		TO237		3	SMC VERSION OF THROUGH-HOLE PART

SO13		---		6	ROHM EMT-6, UPC TO6

SO14		D²PAK/TO263	5	CASE 418B/LINEAR TECH, TO 263-5

SO15		---		8	THAT 2252 MODIFIED FOR SMC, LIE ON SIDE

SO16		SOT143	4	PHILIPS SOT143 WIDE PIN 1 ON LT

SO17		SOT23		5	Count ccw (123)(4x5) LTC-S5 SMV(SSOP5-P) DBV

SO18		SOT143	4	NEC PKG 39 WIDE PIN 1 RT, NE02139 SEE MECH

SO19		D²PAK/TO263	2+	SOT7 OPTIMIZED FOR 130/70(0-80) PAD BTW LEGS

SO20		SOT363	6	SC70-6

SO21		EMT3		3	Rohm EMT-3, DAN222 type

SO22		SMT6		6	Rohm SMT-6, SOT23-6, Siliconix TSOP 6

SO23		UMT6		6	Rohm UMD-6, UMT-6, SC70-6

SO24		D²PAK/TO263	3	Tabless SMT TO-220A

SO25		---		5	Macom ETC1-1-13 SM22 transformer

SO26		SOT343	4	Rotated 45 degrees, manual pad/mask, pads count cw

SO27		SOT343	4	Pads count cw

SO28		SOT353	5	SC70-5

SO29		SOT23		6	Count ccw (123)(456) TK11950 type

SPxx - PLCC/QFP/PGA

SYMB	TYPE	PINS	SCKT	MET	DESCRIPTION / ALIAS 	                                   _

SP1	PLCC	28

SP2	PLCC	44

SP3	PGA	68

SP4	PGA	84

SP5	PGA	128

SP6	QFP	160		l	SCSI CONTROLLER

SP7	PLCC	28			WIDE LAND PATTERN

SP8	PGA	179

SP9	PGA	114

SP10	PLCC	68

SP11	PLCC	84

SP12	QFP	44			Z80A QFP

SP13	LCC	44

SP14	PLCC	44	l

SP15	LCC	20			UC2825 CERAM PWM

SP16	PLCC	20			UC2825 PLASTIC PWM

SP17	PLCC	32			EPROM

SP18	PLCC	32	l		EPROM

SP19	PLCC	28	l		TMS370C710FNA

SP20	QFP	208		l	SUN PCMCIA CONTROLLER

SP21	PLCC	68	l

SP22	PLCC	44	l		Through hole socket

SP23	PLCC	28	l		Through hole socket

SP24	PLCC	44	l		SMC 44PLCC W/ALIGN PINS, ST PLCC-44-F-A

SP25	PLCC	32	l		SMC 32PLCC W/ALIGN PINS, ST PLCC-32-F-A

SP26	QFP	20			MiniCircuits KKK-155 (YSW-2-50DR)

SP27	PLCC	68	l		Through hole socket

SP28	QFP	100		l	TMS320C TQFP

SP29	PLCC	44			ET BC4-44-Spacer-G/B ET emulator adapter

SP30	QFP	24			Motorola FTA

SQxx - SMC X STYLE TRANSISTOR/BIG RF PKG

SYMB	PADW	PADL	DESCRIPTION / ALIAS                                                                         _

SQ1	040	090	NEC OUTLINE 35 STD PADS, NE02135

SQ2	040	060	HP CASE 84/85 PLASTIC, INA-03184 W/HOLE, AVANTEK CASE 84

SQ3	050	090	NEC OUTLINE 35

SQ4	080	100	NEC OUTLINE 35

SQ5	---	---	HP CASE 76 CERAMIC, MGA-86576

SQ6	---	---	HP CASE 86, HP MSA-0986

SQ7	---	---	FLL2001B-2 GAAS-FET, LARGE (13530)

SQ8	---	---	FLL55MK GAAS-FET, SMALL (13530)

SQ9	---	---	145 MIL PLASTIC "B", HP 05 PLASTIC, MSA-1105

SQ10	---	---	WIDE LAND MICRO X

SQ11	---	---	NARROW LAND MICRO X

SQ12	---	---	MEDIUM LAND MICRO X

SQ13	---	---	MINI CIRCUITS AF320 QUAD X ATTENUATOR PAT-X,

SQ14	---	---	HP 100 MIL STRIPLINE PKG

SQ15	---	---	2304CB BPOLAR DRV XSTR, 13504-Q1

SQ16	---	---	AM81720-12 BPOLAR XSTR, 13504-Q2

SQ17	---	---	NEZ-4450-4D XSTR, NEZ-6472 series

SQ18	---	---	NEZ-4450-15D XSTR

SQ19	---	---	Fujitsu FLL120MK

SQ20	---	---	Fujitsu FLL357ME

SQ21	---	---	Stanford-Microdevices SGA5286 large pad layout (BMS)

SQ22	040	060	Stanford-Microdevices SGA5286 minimum pad layout (BMS)

SQ23	---	---	IXYS miniblock mosfet, with bottom and through pads (PCB mount)

SQ24	---	---	SQ23 without bottom pads, all silk moved to top (Chassis mount)

SRxx - SMC DIODE/RECTIFIER

SYMB	Round	BodyX	BodyY	LeadX	LeadY	DESCRIPTION / ALIAS                                                   _

SR1		190	120	210	070	DL41, D64 CHIP DIODE, 10MQ090, SMA

SR2						403A-01 CHIP DIODE

SR3		080	050	080	050	0805 LED, CITIZEN ELEC CL-170G-CD

SR4	l					SEMITEC CRD F-TYPE ROUND BODY

SR5	l					SA13 MINIMIZED AREA MOD 1N4000 DIODE

SR6	l					MOT CASE 17-02 AXIAL MOD, 1N5352B 5W ZENER

SR7	l					SOD123 ROUND BODY

SR8	l					LL-34 SOD ROUND BODY

SR9						MP2X2718 MICROWAVE DIODE PIN SWITCH (13572)

SR10	l	150	062d	1501	062d	SOD F-TYPE, RDI ROUND GLASS MINI-MELF, DL35

SR11		120	062	160	030	SOD123, Diodes (Digikey) SD101/SD103 diodes

SR12		080	020	110	060	SOD323, Diodes (SCM mic2)

SR13		120	060	120	060	1206 CHIP LED

SR14		190	150	220	090	DO214AA, SMA/SMB

SR15		280	250	320	130	SMC

STxx - TO CASE(THROUGH HOLE)

SYMB	TOx	PINS	LDX	LDY	LDC	LCFG	MNT	DESCRIPTION / ALIAS           _

ST1	3	---	---	---	---	---	V/R	MASK OVERLAY

ST2	3	2+	---	---	---	---	H

ST3	3	2+	---	---	---	---	V

ST4	3	2+	---	---	---	---	V/L

ST5	5	3	---	---	200	---	V	STD, RP1105 SAW RSNTR

ST6	5	8	---	---	300	---	V	WIDE

ST7	5	10	---	---	300	---	V	WIDE

ST8	8	4	----	---	300	---	V	TO8U/T, AVAN UTO/AM162

ST9	18	3	---	---	100	---	V

ST10	18	3	---	---	150	---	V

ST11	18	4	---	---	100	---	V

ST12	18	4	---	---	150	---	V

ST13	18	2	100	---	---	ST	V	NORMAL

ST14	18	2	150	---	---	ST	V	WIDE

ST15	92	2	150	---	---	ST	V	NORMAL

ST16	92	3	---	---	150	---	V	

ST17	92	3	100	---	---	ST	V

ST18	126	3	100	---	---	ST	H	LONG LEADS

ST19	247	3	200	---	---	ST	V	TO218=247, 218=070 HOLE

ST20	247	3	200	125	---	CCW	V

ST21	247	3	215	---	---	ST	V

ST22	247	3	225	---	---	ST	V

ST23	247	3	225	150	---	CCW	V

ST24	247	3	225	150	---	CW	V

ST25	247	3	250	---	---	ST	V

ST26	247	3	275	---	---	ST	V

ST27	220	2	200	---	---	ST	V	NORMAL

ST28	220	3	100	---	---	ST	V	NORMAL

ST29	220	3	100	---	---	ST	H	150 O/S IN FRONT/BODY

ST30	220	3	125	---	---	ST	V	150 O/S IN FRONT/BODY

ST31	220	3	100	200	---	CCW	V	VDE SPACING/PRI SIDE

ST32	220	5	075	100	---	STG	H	100 FROM BODY

ST33	8	12	400	400	---	SQ	V	PIN SQUARE

ST34	220	5	075	075	---	STG	V

ST35	247	3	225	---	---	ST	H	225 FROM BODY

ST36	5	8	---	---	200	CCW	V	H08C

ST37	8	4	---	---	---	---	V	VARIL TO-8, 90 DEG ROT

ST38	8	4	---	---	---	---	V	VARIL TO-8, 135 DEG ROT

ST39	5	8	---	---	230	CCW	V	H08D, FLUSH MOUNT

ST40	5	10	---	---	230	CCW	V	FLUSH MOUNT

ST41	220	5	075	---	---	ST	H	SMC OF TO220-5

ST42	92ua	3	070	---	---	ST	V	Hall effect sensor

ST43	247	3	225	---	---	ST	H	T03 type tab, Q40 alternister

SVxx - VCO BOX

SYMB	SMT	DESCRIPTION / ALIAS                                                                                      _

SV1	l	ZCOMM C600T

SV2		ZCOMM C804, V6049002

SV3		SV2 MOUNT ON BOTTOM, SHOW FROM TOP

SV4	l	ZCOMM C707 LARGE

SV5	l	ZCOMM V605, V900ME01, V600MC05, Emhiser MS PKG, 1/2" universal foot

SV6	l	ZCOMM 300 SQ SUB-MINI

SXxx - MISC PARTS

SYMB	DESCRIPTION / ALIAS                                                                                                  _

SX1	SIMM SOCKET

SX2	SIMM SOCKET

SX3	D0004E MARKI DOUBLER, D0002LE (0.60 x 0.32)

SX4	MSI AMP, Z CONFIGURATION C-BAND

SX5	MSI AMP, Z CONFIGURATION S-BAND

SX6	1/2 SIZE MARKI DOUBLER

SX7	T95202 MARKI TRIPLER

SX8	Computer conversions TDSM910 Resolver-digital encoder, all 28 PINS + key

SX9	C-BAND LP/HP PIN SWITCH MODULE, ALLIANT (SMALL ONE, NOT USED)

SX10	TO5-10 ON VERTICLE BOARD, T05 HORIZ MOUNT

SX11	6 PAD T05 CONVERTER BOARD, SEISMOGRAPH

SX12	MSI MSH-4226501-SW LP/HP PIN SWITCH MODULE, ALLIANT

SX13	Combiner from old PA design

SX14	MSI MSH-5218701-DI amplifier module, C-Band (13689)

SX15	Fujitsu FMM5027VJ, Case VJ 6 pin quad flat pack SMC

SX16	Emhiser micro tech ELCO-TLS-xxx 0.35 square SMC VCO

See MMxxx for Marki mixers

See MPxxx for power supplies/converter modules

CCxx - CHIP COMPONENTS

SYMB	ENG	MET	POL	MIL	DESCRIPTION / ALIAS                                                _

CC1	0805	2012			RC575, RCW575, MS5342K06B, 500R15

CC2	1206	3216	l		AVX-A, TAJ-A, KEMET T491A, PCS-Y

CC3	1206	3216

CC4	0504	---			500R11

CC5	0505	---			RC550, RCW550, UJJ

CC6	0806	---			ATC100A, 700A, CY81, CDR11

CC7	0806	---		l

CC8	0504	---		l	500R11

CC9	0505	---		l	UJJ

CC10	0805	---		l	500R15

CC11	0907	---		l

CC12	1005	---		l	RC5100, RCW5100

CC13	1010	---			RC1100, RCW1100

CC14	1010	---		l	RC1100, RCW1100

CC15	1206	3216		l

CC16	2917	7343	l		AVX-D, EIA-D, TAJ-D, AVX-TPS-E(7343H)

CC17	1411	3528	l		AVX-B, EIA-B, TAJ-B, PCS-X, PCS4225

CC18	2413	6032	l		AVX-C, EIA-C, TAJ-C, KEMET T491-C,PCS-C

CC19	1210	3225			LQH3C/N, LQN2A, 380NB/LB/KB/HB, 32CS, 500S41

CC20	1206	3216	l	l	AVX-A, TAJ-A

CC21	1210	---		l	500S41

CC22	1311	---			ATC-100B, 175B, 700B, CY83, CDR13

CC23	1311	---		l	ATC-100B, 175B, 700B, CY83, CDR13

CC24	1812	4532			LQH4N, 43CS, 43FS, 435,PM40

CC25	---	---	l		SANYO 6.3mm DIA SMC ELECTROLYTIC,NIC 6.3mm (Case-D)

CC26	1836	---			3 WIDE 1812 STACK

CC27	2917	---	l		KEMET T491-D, MATSUO-D

CC28	2923	---	l		MATSUO-E, 267-M

CC29	3333	---	l		NICHICON 8mm DIA SMC ELECTROLYTIC (Case-E/F)

CC30	2215	---			BMS HANDWOUND COIL

CC31	1990	---			BMS HANDWOUND COIL VCO-STD

CC32	2214	---	l		49BC-H, CWR06-F, BLUE-CHIP

CC33	2711	---	l		TAZ-G

CC34	0603	1608

CC35	2018	---			Coilcraft 1812CS/LS/CX series

CC36	1005	---

CC37	1206	---			MODIFIED TO PASS 3 10MIL TRACKS

CC38	1211	---			COILCRAFT 1008CS/LS/CX SERIES

CC39	5320	---			EPSON MA505 CRYSTAL

CC40	2012	---

CC41	2220	5750			NL565 CHOKE

CC42	---	---			Coilcraft BxxT spring choke

CC43	3224	---			PANASONIC TYPE-CF ZNR

CC44	0805	2012			MODIFIED TO PASS 1 20MIL TRACK, (R ONLY,NO C)

CC45	1913	---			PANASONIC FILM CAP

CC46	0805	2012			PA VERSION, SMALL PADS-TOP ASM ONLY

CC47	1206	3216				"			"

CC48	1812	4532				"			"

CC49	1206	3216				"			"    CHOKE SYMBOL

CC50	2413	6032	l			"			"

SYMB	ENG	MET	POL	MIL	DESCRIPTION / ALIAS                                                _

CC51	1206	3216			INDUCTOR SYMBOL ON 1206 FOOTPRINT

CC52	1813	---			TDK NLF4532,NFM41R 3 PAD EMI FILTER

CC53	1804	---			CTS NTF SERIES 3 PAD EMI FILTER

CC54	---	---			COILCRAFT AxXT SPRING COIL CHOKE

CC55	3820	---			LITTLE FUSE NANO-2 FUSE HOLDER

CC56	1305	---			NFM40R SERIES 3 PAD EMI FILTER

CC57	---	---			NFM51R SERIES 3 PAD EMI FILTER

CC58	---	---			595D, CASE-R ORANGE TANTALUM

CC59	---	---			300LS SMC CHOKE

CC60	---	---			SANYO 8MM DIA SMC ELECTROLYTIC

CC61	---	---			CHOKE+Z-BEAD, 13504-L1/L2

CC62	1311	---			PA VERSION, SMALL PADS T/A ONLY ATC-100/700B

CC63	0806	---				"	"			ATC-100/700A

CC64	1812	---			UNIVERSAL PAD, 0805 TO 1812

CC65	4924	---			Delevan 4922 series power inductor

CC66	2512	6332			IMS RC2512 2W SMC RESISTOR

CC67	1813	---			4750470 IND, WIDE PAD FOR NL45-3232-XXX

CC68	---	---			Coilcraft 0906 spring choke

CC69	---	---			Coilcraft 1606 spring choke

CC70	8431	---			Delevan 8532 series power inductor

CC71	0805	---			Choke symbol on 0805 footprint

CC72	---	---	l		CC58 tantalum (Case-R) on edge

CC73	0805	---			Utilicom 0805

CC74	---	---			Isaplan SMV kelvin connected sense resistor

CC75	---	---	l		4mm SMC radial (Case-B)

CC76	2525	---			JW Miller PM54 inductor, round footprint

CC77	0603	---			Reduced outline version of CC34

CC78	0402	---			TES standard 0402

CC79	1206	---			Panasonic EXB-38Vxxx 4 element isolated network

CC80	---	---			JW Miller PM105S inductor, round footprint

CC81	1206	---			CTS 742 series 4 element, std footprint

CC82	1206	---			CTS 742 series 4 element, shrink footprint CC79 overlay

CC83	2923	7257			Garret-E, not AVX-E which is tall D (Garret-D3)

CC84	---	---			UNI-PAC UP2B-xxx power inductor

CC85	---	---			Coilcraft 1812WBT SMC transformer

CC86	1008	2520			Garret KQ1008 inductor

CC87	---	---	l		16mm SMC radial with 2 dummy side pads (Case-J16)

CC88	---	---			Coiltronics 3316 power inductor

CC89	2010	---			2010 chip resistor

CC90	1305	---			NFM3212R SERIES 3 PAD EMI FILTER

CC91	---	---	l		10mm SMC radial (Case-G)

CC92	---	---			Central CTDO3316 series inductor / PM3316

CC93	---	4532			TDK NLFC453232T-xxx inductor

CC94	1515	---			Coilcraft 1008PS

CC95	---	---			Coilcraft DS1608

CC96	---	---			Panasonic EZA-ST R/C network, 10 pin

CC97	---	---			JW Miller PM5022 inductor

CC98	---	---			Toko D124C inductor

RAxx - AXIAL RESISTOR

SYMB	BLEN	BWID	LDSP	DESCRIPTION / ALIAS 	                                                _

RA1	250	100	400	1/4W, RN55

RA2	170	100	300	1/8W, RN50

RA3	250	100	300	1/4W, RN55

RA4	550	200	700	1W CARB, RN65

RA5	250	100	450	IT3 BIFURCATED ON ENDS

RA6	410	170	600	JW MILLER CHOKE

RA7	450	150	650	TOP SILK ONLY, 476-0700-00 0.1 1W RES

RA8	1050	350	1350	Mouser VL5 5 watt resistor

RA9	750	250	900	Delevan 2474 series chokes

RA10	600	200	800	1W CARB, RN65

RNxx - SIP R-NETWORKS/JUMPERS (0.100)

SYMB	PINS	DESCRIPTION / ALIAS 	                                                                         _

RN2	2	2 pin jumper

RN3	3

RN4	4

RN5	5

RN6	6

RN7	7

RN8	8

RN9	9

RN10	10

RPxx - POTS

SYMB	SMC	DESCRIPTION / ALIAS 	                                                                         _

RP1		72PR, 63P, 3386P, 362Y, E2A

RP2		72XR, 63X, 3386X, 360S, E4A

RP3		3299X, 64X, 67X, 3296X, ?

RP4		3299P, 64X, 67X, 3296X, ?

RP5		3299W, 64W, 67W, 3296W

RP6		3260W, 3262W, 3266W, RJ26W, RJR26W, 64WR, 98W, 74W

RP7		3329P, 3329U, 62MR, ET50P, 75P, SVxxx1, 8014EMBxxx, 171-xx, 181-xx

RP8		3386S, 63S, 72SR

RP9		MOUSER 32AAxxx 8MM HORIZ OPEN FRAME

RP10		3323P, 67P, 82P, A10, 25P, W780M

RP11		3262P, RJ26P, RJR26P

RP12		3262X, 3266X, RJ26X, RJR26X

RP13	l	BOURNS 3314G

RP14	l	32GGW 1/4" TOP ADJ MOD

RP15		BOURNS 3009Y

RP16		TOCOS 6W ALIGNED HORIZ/VERT

RP17		TOCOS 6W ALIGNED DIAG

RP18		CTS 450 SERIES GANGED POT

RP19		MISC GANGED POT

RP20		MOUSER 31VQ SERIES 3/8" PC MOUNT

RP21	l	BOURNS 3335W 5MM SQ GULL-WING

RP22	l	PANA EVM-7JSX 3MM, BI 22A

RP23	l	BOURNS 3335A

RP24	l	BOURNS 3269P, MEPCO ST5P-xx

RP25	l	MOUSER 328-45-xx

RP26	l	Bourns 3314J vertical adjust

RP27	l	Bourns 3314S horizontal adjust

RP28	l	MEPCO ST5W-xx 11 TURN

RP29	l	RP22 FOR P/A TOP ASSY ONLY

RP30	l	MEPCO ST5A-xx

RP31	l	BOURNS 3335X

RP32		MOUSER MEC-4290W 13T POT, SEISMOGRAPH

RP33		Mouser 322-5800-xx open fram pot, NSM

RP34	l	Bourns 3224W 11 turn pot, vert adjust, 200x150

RP35	l	3262P, RJ26P, RJR26P Leads formed to surface mount

RP36		3299Y

RP37	l	Bourns 3214J side adjust

RS/Txx - SHUNTS/THERMISTERS

SYMB	LINE	LENG	MV/A	WT	DESCRIPTION / ALIAS                                                _

RS1	12/10	350	20	1

RS2	16/14	350	15	1

RS3	22/20	350	10	1

RS4	42/40	800	10	1

RS5	22/20	3000	50	1.5

RS6	10/8	350	50	1

SYMB	BLEN	BWID	LX	LY	DESCRIPTION / ALIAS                                                _

RT1	550	250	350	200	Panasonic KC015-021

RT2	800	260	330	200	Keystone (Mouser) 527-CL40

CAxx - AXIAL CAPS (RADIALS ON SIDE)

SYMB	BLEN	BWID	LDSP	A/R	POL	DESCRIPTION / ALIAS                                    _

CA1	1060	510	1260	A	l

CA2	450	200	080	R	l

CDxx - DIP POTTED CAP, FILM, ALLIGNED PINS

SYMB	BLEN	BWID	LDSP	POL	DESCRIPTION / ALIAS                                                _

CD1	300	100	200		CK05, CK06

CD2	700	200	600		DIGIKEY/PANA P4638/ECQ-U2A563MV

CD3	1100	400	900		DIGIKEY/PANA EF4105/ECQ-E4105KF

CD4	270	190	120		DM05 SINGLE/DOUBLE COAT

CD5	200	100	100		Mouser UEC/Z case 904

CD6	200	150	100		Mouser UEC/Z case 905

CD7	350	100	250		NSM 1000/2000PF ceramic

See CK for disk caps (offset pins)

CKxx - DISK CAP, OFFSET PINS

SYMB	BLEN	BWID	LDSP	LOSET	DESCRIPTION / ALIAS                                                _

CK1	350	150	250		Mouser CD50x, 8.5x6.4mm

See CD for disk caps (alligned pins)

CRxx - RADIAL CAP

SYMB	DIA	LDSP	ROT	POL	DESCRIPTION / ALIAS                                                _

CR1	700	300	0	l	BMS 150-0277-01

CR2	500	200	0	l

CR3	200	100	0	l	Sprague case A tantalum, 199D, 5mm electrolytic

CR4	1000	400	0	l

CR5	1000	400	45	l

CR6	900	400	0	l	BMS 220/250 BLACK NIC

CR7	360	110	0		MOUSER 434-02-XXXJ HI-Q CHOKE

CR8	350	150	0	l	XICON 8MM ELEC, TES CAPS!

CR9	400	200	0	l	Xicon 10mm electrolytic

CR10	250	100	0	l	Xicon 6.3mm electrolytic

CR11	300	200	0	l	Tant, 100uF 20V

CR12	650	200	0	l	NSM 2200/25V

CVxx - TRIMMER CAP

SYMB	SMC	DESCRIPTION / ALIAS 	                                                                         _

CV1	l	GARRET/PHILIPS 2807S SERIES

CV2		JOHANSEN 27281SL

CV3		JOHANSEN 27285 ASSEMBLE FROM BOT FOR TOP ADJUST

CV4	l	TZB04

CV5		FROM 1707500000 PWB

CV6	l	MISC

CV7	l	SPRAGUE GKG SERIES, DIGIKEY 20xx SERIES

CV8		JOHANSEN JMC2400-x

CV9	l	VOLTRONICS JZxxx SERIES

CV10	l	TOP ASSY PA VERSION, JOHANSEN 27273

CV11	l		"	"	JOHANSEN 27261

CV12		Johansen JMC-5751 3 hole horizontal adjust

CV13	l	Johansen 9401-1 cap for filter

CV14	l	CV13 with round pads @ 45°

CV15	l	CV13 with round pads @ 112.5°

CV16	l	CV13 with round pads @ 67.5°

CYxx - CRYSTAL (NON-DIP)

SYMB	SMC	DESCRIPTION / ALIAS 	                                                                         _

CY1		HC49U, HC18U STANDARD HORIZONTAL MOUNT, LONG LEADS

CY2		CY1 WITH SHORT LEADS

CY3		HC49S VERTICAL MOUNT

CY4		HC45/U-2, HC52U, ICM (FM-5) HORIZONTAL MOUNT

CY5		CY4 WITH TOP END GROUND PAD

CY6		CY1 WITH TOP END GROUND PAD

CY7	l	EPSON MA-505 2 LEAD

CY8	l	MOUSER 332SML SERIES

CY9	l	ECS ECX-3A SERIES

CY10	l	CY1/CY6 MODIFIED TO SMC, 050 THICK CAN HORIZ

CY11		MHE BIG CAN OVEN CRYSTAL

CY12	l	EPSON MA-506 4 LEAD

CY13	l	Utilicom multipackage clock osc

CY14	l	Epson MC206 (6GP sleep)

CY15	l	Epson C-001R / DK SE3201

SEE MFxx FOR SAW FILTERS

MFxx - FILTERS

SYMB	SMC	PRINT	BLEN	BWID	DESCRIPTION / ALIAS                                                _

MF1			---	---	BMS BANDSTOP

MF2			---	---	ALLEN AVIONICS F4317-SP2

MF3			---	---	MATTHEY P405 PVE SIP

MF4			750	450	LARK MC2245-600-4MM, CHECK MECH, MC710-45-4MM

MF5			1000	450	LARK MC610-40-4MM, CHECK MECH

MF6			300	100	TOKO 3 LEG CERAM

MF7	l		---	---	ALLEN AVIONICS VIDEO

MF8	l	l	---	---	BMS 4GHZ DOWN STEP BAND PASS

MF9	l	l	---	---	BMS 4GHZ UP STEP BAND PASS

MF10			870	500	851475, 851848 SAW

MF11			---	---	TOKO CBW DOUBLE TUNED HELICAL

MF12			---	---	TOKO CBT TRIPLE TUNED HELICAL

MF13	l		530	260	SAWTEK 854661 CASE SMP-53 SAW

MF14			750	450	MF4 with I/O top pads removed and top ground fill

MF15	l		1050	480	Integrated microwave 6mm/4-S12 ceramic filter

SEE CCxx FOR POWER LINE SMC FILTERS

SEE ALSO SRFxx

MHxx - HEADERS/COIL MOUNTS

SYMB	SMC	PINS	DESCRIPTION / ALIAS                                                                         _

MH1	l	8	TM375 normal fit, 0.400 mag space

MH2	l	8	TM375 tight fit, MH1

MH3	l	4	4 pin version MH1

SEE ALSO MTxx

MIxx - ISOLATOR/CIRCULATOR

SYMB	LOAD	SIZE	DESCRIPTION / ALIAS                                                                         _

MI1	EXT	500	Mica DNF1300-TO600

MI2	INT	500	Mica DNF1300-TO600

MI3	EXT	500	Mica DNF1300-TO600 with resistor cut-out

MI4	EXT	500	Mica DNF1300-TO600 with resistor on plate cut-out

MMxx - MIXER/SPLITTER/COMBINER

SYMB	SMC	DESCRIPTION / ALIAS                                                                                      _

MM1	l	M/C RMS SERIES, CASE TT100/146/240/241

MM2		M/C CASE TUF-B02/13

MM3	l	M/C CASE QQQ130, LRPS-2-11

MM4	l	MARKI MI-0206LE MIXER (WIRE CONNECT)

MM5	l	M/C CASE BH

SEE SXxx FOR DOUBLERS

MPxx - POWER TRANSFORMER

SYMB	DESCRIPTION / ALIAS                                                                                                  _

MP1	IF-XX-XX SERIES

MP2	TRIAD F-119X

MP3	CALEX UW SERIES POWER CONVERTER

MP4	ASTRODYNE MTSA-1011 AC-DC MODULE, BMR-120

MP5	Signal split tran DST-7-24 power transformer

MP6	Magnetek F24-250 6VA PC mount single primary

MP7	Talema 1.6VA torroid pcb mount, TE70000 (Digikey)

MQxx - CORE SETS/COMPONENTS

SYMB	TYPE	SIZE		DESCRIPTION / ALIAS                                                             _

MQ0	PQ	2016-2020	0 DEG ROTATION

MQ1	PQ	2016-2020	18 DEG ROTATION

MQ2	PQ	2016-2020	36 DEG ROTATION

MQ3	PQ	2016-2020	54 DEG ROTATION

MQ4	PQ	2016-2020	72 DEG ROTATION

MQ5	PQ	2016-2020	10 DEG ROTATION

MQ6	PQ	2020		BARE CORE

MQ7	---	---		13398 POWER XFMR #1

MQ8	PQ	2016-2020	0 DEG ROTATION

MQ9	---	---		13398 POWER XFMR #2

MQ10	PQ	2016-2020	BOBBIN HEADER THROUGH HOLES

MQ11	PQ	2016-2020	J-LEADED BOBBIN

MQ12	LP	22/13		Bobbin, mounted through board (Quasi-SMC)

MQ13	LP	22/13		Tight fit MQ12

MSxx - SPECIALIZED MAGNETICS

SYMB	DESCRIPTION / ALIAS                                                                                                  _

MS1	TOKO 5CT FIXED INDUCTOR, 5CD FOOTPRINT (312AN)

MS2	MAGNETEK SP70 4 PIN XFMR, ON SIDE.  560-0059-01 SEE MECH

MS3	PANASONIC ELF-18D624F TYPE 600H, D/K PLK-1036 EMI CHOKE

MS4	MS2 NORMAL MOUNTING

MTxx - TORROID/POT/RM/SQUARE/CHOKE

SYMB	TYPE	PTRN	DIA	SIZE	DESCRIPTION / ALIAS

MT1	POT	---	905	---	MICRO/M VTM421-02 mount, A Triana NTSC vid filter

MT2	PQ	2016	---	---	SRT power supply

MT3	RM	1812	---	---	Magnetics 6 pin with core, mirror driver

MT4	RM	1812	---	---	Magnetics 6 pin bobbin only

MT5	TOR	---	875	---	JWM 5701-5705, Mouser 542-xxxx

SEE ALSO MHxx/MQxx  CCxx for SMC inductors

MVxx - VARIABLE INDUCTOR

SYMB	SMC	PINS	CONN	DESCRIPTION / ALIAS                                                             _

MV1	l	5	1-3	TOKO 5CCA, 5CCB, 5CCD, 5CCL, 5CE, 614BN

MV2	l	6	1-3	TOKO 5CCE, 5CCF, MV1 WITH 6 PINS, A638AN

MV3	l	5	1-3	TOKO 5CD

MV4		5	1-3	TOKO 10EZ/EZC/EZH, BTKXNS-T

MV5		6	---	TOKO 10K/KC/SE, MV4 WITH 6 PINS

MV6		2	---	TOKO MC122

MV7		4	1-4	CAMBION 558-1192-xx BIG CENTER POST

MV8		5	1-3	TOKO 7P, 7PA, 7PH

MV9		5	1-3	MV3 WITH TAB HOLE FOR CADDELL-BURNS 6740 SERIES

MV10	l	4	1-3	TOKO 5CB

IAxx - ADAPTERS/SOCKETS

SYMB	FROM	PINS	TO	PINS	DESCRIPTION / ALIAS                                                _

IA1	SOIC	14	SOIC	16	FIX FOR SOIC LAYOUT 150 BODY

IA2	DIP	28	HDR	28	1.25mm ADAPTER FOR RDI

IA3	TSOP	40	VSOP	40	STRIP FOR BRUCE EPROM

IA4	TSOP	40	DIP	40	10x20 TSOP PROGRAMMER SOCKET

IBxx - BARRIER STRIP/TERM BLOCK

SYMB	PINS	PITCH	ROW	DESCRIPTION / ALIAS                                                             _

IB1	9	375	1

IB2	24	200	1

IB3	10	200	1

IB4	12	200	1

IB5	8	375	2	Standard terminal block w/screw in pcb pins, Mouser 15BB00-8

IB6	8	200	1	MOUSER 506-2MV08

IB7	2	200	1	MOUSER 506-2MV02

IB8	12	200	1	MOUSER 506-2MV12

IB9	2	375	1	Mouser 506-6PCV-02

IB10	3	375	1	Mouser 506-6PCV-03

IB11	4	375	1	Mouser 506-6PCV-04

IB12	5	375	1	Mouser 506-6PCV-05

IB13	6	375	1	Mouser 506-6PCV-06

IB14	7	375	1	Mouser 506-6PCV-07

IB15	8	375	1	Mouser 506-6PCV-08

IB16	9	375	1	Mouser 506-6PCV-09

IB17	10	375	1	Mouser 506-6PCV-10

IB18	12	375	1	Mouser 506-6PCV-12

IB19	14	375	1	Mouser 506-6PCV-14

IB20	15	375	1	Mouser 506-6PCV-16

IB21	4	200	1	Mouser 506-2MV04

ICxx - RELAY/FUSE/SWITCH

SYMB	TYPE	SMC	DESCRIPTION / ALIAS                                                                         _

IC1	SW		DPDT TOGGLE

IC2	SW	l	SPDT TOGGLE

IC3	SW		SLIDE VOLTAGE SELECTOR

IC4	SW		ROTARY VOLTAGE SELECTOR

IC5	SW		KEYBOARD KEY

IC6	SW		RT ANGLE IC3

IC7	SW		DPDT TOGGLE

IC8	SW		DPDT TOGGLE

IC9	SW		THUMBWHEEL

IC10	SW		THUMBWHEEL

IC11	SW		THUMBWHEEL

IC12	SW		THUMBWHEEL

IC13	ROT		TO5 ROTARY

IC14	ROT		MOUSER 4P3T ROTARY

IC15	SW		KEYBOARD KEY

IC16	SW		6P2T SLIDE

IC17	SW		4P2T RT ANGLE SLIDE

IC18	SW	l	MEMBRANE PATTERN, BMS

IC19	ROT		BINARY ROTARY, DIP 5 pin, not NSM

IC20	SW		MOUSER 101-0010 SNAP PCB SWITCH

IC21	FUSE

IC22	FUSE

IC23	FUSE	l

IC24	RLY		SMALL RADIO SHACK TYPE

IC25	RLY		14 PIN DIP

IC26	RLY		SIP REED

IC27	RLY		DIP REED

IC28	RLY		SIP REED

IC29	RLY		ODD REED

IC30	RLY		TELEDYNE 8 PIN TO5

IC31	RLY		HI AMP AUTO

IC32	RLY		KRACO

IC33	RLY		KRACO

IC34	RLY		SSR OUTLINE

IC35	SW		Alco MSS-4200 4P2T slide switch, Vert IC17

IC36	ROT		Hex rotary switch, 6 pin NSM type

IC37	RLY		120VAC relay

IC38	RLY		Small plug in relay, W7PCX-x, mouser 528-720-x (NSM)

IC39	SW		NKK YB-25 2P2T push button with lamp

IC40	SW		NKK YB-25 2P2T push button with lamp, no copper in pad centers

IC41	RLY		Potter-Brumfield (Seimens) RTD140xx/RTE240xx relays

IC42	SW		4P2T toggle switch, probably the one I used on the 6GP battery charger

IC43	SW		CanTherm F11xxx151ZA0060 thermal cutoff

IDxx - D-SUB CONNECTORS

SYMB	PINS	SEX	H/V	M/H	TYPE		DESCRIPTION / ALIAS                       _

ID1	37	S	H	2	DC-37S-R	AMP 745784-4

ID2	37	P	H	2	DC-37P-R	AMP 747252-4

ID3	25	S	H	4	DB-25S-R

ID4	25	P	H	4	DB-25P-R

ID5	25	S	H	2	DB-25S-R	AMP 745783-4

ID6	25	P	H	2	DB-25P-R	AMP 747238-4

ID7	9	S	V		DE-9S

ID8	9	P	V		DE-9P

ID9	15	S	V		DA-15S

ID10	15	P	V		DA-15P

ID11	25	S	V		DB-25S

ID12	25	P	V		DB-25P

ID13	37	S	V		DC-37S

ID14	37	P	V		DC-37P

ID15	26	P	V				HI DENSITY 3 ROW

ID16	62	P	V				HI DENSITY 3 ROW

ID17	25	S	H		DB-25S-R	AMP 745992-4/LONG BODY

ID18	50	S	H	2			HI DENSITY 4 ROW/AMP 74726-X50

ID19	15	P	H	2	DA-15P		AMP 747236-4

ID20	15	S	H	2	DA-15S		AMP 745782-4

ID21	9	P	H	2	DE-9P		AMP 747250-4

ID22	9	S	H	2	DE-9S		AMP 745781-4

IExx - ELASTOMER CONNECTOR

SYMB	PINS	PAD	PITCH	DESCRIPTION / ALIAS                                                             _

IE1	16	020	030	SAMTEC ELC-S

IE2	33	020	030	SAMTEC ELC-S

IE3	22			?

IECxx - EDGE CONNECTOR

SYMB	PINS	PAD	PITCH	DESCRIPTION / ALIAS                                                             _

IEC1				BMR-120 motherboard connector

IFxx - FEED THROUGH

SYMB	DESCRIPTION / ALIAS                                                                                                  _

IF0	MURATA 1234-000 PAD FOR BODY AND CENTERPOST WIRE

IF1	MURATA 1234-000 BODY PAD ONLY

IJxx - JUMPERS

SYMB	PINS	SPC	SMC	DESCRIPTION / ALIAS                                                             _

IJ0	2	---	l	HALF CIRCLE SOLDER JUMPER

IJ1	2	100

IJ2	3	100

IJ3	20	---	l	IJ0-10 POSITION

IJ4	2			ULINE JUMPER, 289-0013-06 POSTS

IJ5	3			IJ4 WITH JUMPER STORAGE PIN

IMxx - MISC CONNECTORS

SYMB	PINS	H/V	DESCRIPTION / ALIAS                                                                         _

IM1	40	H	4 row RDI connector

IM2	40	V	IM1

IM3	6	H	Amp #520258-3 phone jack, Digikey A9073

IM4	5	H	Switchcraft 57PC5F DIN, Mouser 161-0503, Digikey #CP-2350

IM5	10	V	Molex A5566-N (NSM), Huey triac interface board type

IM6	5	H	Mouser 161-3504 stereo 1/8 audio jack, NSM

IM7	3	H	Mouser 161-3505 mono 1/8 audio jack, NSM

IM8	6	V	Molex A____-N (NSM)

IM9	3	V	Molex receptacle

IM10	15	H	MDI power supply connector

IM11	4	H	RDI power connector, LCD B/L

IM12	2	V	Molex receptacle

IM13	16+	H	6GP data connector, 90-58470

IM14	18+	H	Opal data connector

IM15	18+	H	S3/K3 data connector

IPxx - POWER CONNECTORS

SYMB	PINS	DESCRIPTION / ALIAS                                                                                      _

IP1	3	Switchcraft RAPC-722 power jack, Mouser 16PJ031, NSM 162-0001

IP2	2	AA battery holder, Keystone/Mouser #534-092 clips (Minimum 0.70 C-C)

IP3	2	Keystone #106 20mm 1 cell lithium holder (AKOS)

IRxx - RF CONNECTORS

SYMB	H/V	SMC	DESCRIPTION / ALIAS                                                                         _

IR1	H		RADIALL R114-665 SMB RT ANGLE

IR2			JOHNSON 1Q42-0701-801/806 SMA

IR3			MAACOM 2367-xxxx-54 OSMT RECEPTICAL

IR4	V	l	RADIALL R209-406 MMS SERIES

IR5			MOUSER 16PJ092 RCA

IR6	V	l	SUHNER 90 MMCX-S50-0-51

IR7	H	l	SUHNER 90 MMCX-S50-0-51

IR8	H		Suhner 85MMCX-50-0-1 coax connector

IRJxx – TELEPHONE CONNECTORS

SYMB	H/V	SMC	PINS	POS	DESCRIPTION / ALIAS                                                                     _

IRJ1	H		6/6	1	Amp 555165-1 without stops

IRJ2	H		8/8	1	Amp 555164-1 without stops

IRJ3	H		8/8	2	Amp 557560-1 unshielded

IRJ4	H		8/8	3	Amp 557561-1 unshielded

ISxx - BENDIX PTO SHELLS

SYMB	TYPE			DESCRIPTION / ALIAS                                                             _

IS1	PT02E-22-55S 		PCB PARTIAL LAYOUT

IS2	PT02E-22-32S		PCB ADAPTER PATTERN

IS3	PT02E-22-32P		PCB ADAPTER PATTERN

ITxx - TERMINALS

SYMB	PAD	HOLE	SMC	DESCRIPTION / ALIAS                                                             _

IT0				BMS STANDARD SWAGE TERMINAL

IT1	040	---	l	TOP SQUARE

IT2	050	035		T-1R13-G/T PRESS FIT TERMINAL

IT3	140	072		100/070 BIFURCATED TERMINAL

IT4	050	025

IT5	060	025

IT6	080	035

IT7	120	064		BMS STANDARD GOLD PIN (USE IT0)

IT8				1/4 pcb mount quick disconnect

IWxx - WALDOM/MOLEX 2 MM HEADERS

SYMB	PINS	DESCRIPTION / ALIAS 		                                                            _

IW6	6	Molex 53014-0610  (Mating pin housing:  51004-0600)

IW8	8	Molex 53014-0810  (Mating pin housing:  51004-0800)

IW10	10	Molex 53014-1010  (Mating pin housing:  51004-1000)

IW12	12	Molex 53014-1210  (Mating pin housing:  51004-1200)

IW14	14	Molex 53014-1410  (Mating pin housing:  51004-1400)

IHxx - HEADERS/SOCKETS

SYMB	SMC	PINS	C-C	R-R	ROW	R/A	M/F	DESCRIPTION / ALIAS           _

IH1		9	156	---	1	l	M	TOP WALL

IH2		50	100	100	2		M/F

IH3		16	100	100	2		M/F	NUMB A1/2...H1/2

IH4		20	100	100	2		M/F	NUMB A1/2...J1/2

IH5	l	4	100	050	2		M/F	WIRED 3 PIN JUMPER

IH6	l	20	100	050	2		M/F

IH7		2	100	---	1		M	LARGE BASE BACK WALL

IH8		2	100	---	1		M/F

IH9		5	100	---	1		M	LARGE BASE BACK WALL

IH10		3	100	---	1		M	LARGE BASE BACK WALL

IH11		4	100	---	1	l	M/F

IH12		4	100	---	1		M	LARGE BASE FRONT WALL

IH13		5	156	---	1		M	LARGE BASE BACK WALL

IH14		6	156	---	1	l	M

IH15		6	156	---	1		M	LARGE BASE BACK WALL

IH16		6	156	---	1	l	M

IH17		7	100	---	1		M/F

IH18		8	100	---	1		M	LARGE BASE BACK WALL

IH19		9	100	---	1		M	LARGE BASE BACK WALL

IH20		10	100	---	1		M	LARGE BASE BACK WALL

IH21		10	100	100	2		M/F

IH22		11	100	---	1	l	M

IH23		20	100	100	2		M/F	LARGE BASE

IH24		20	100	100	2	l	M	SHROUDED IDC

IH25		20	100	100	2		M	POLARIZED IDC, 4 wall shroud

IH26		20	100	100	2		M	SHROUDED IDC

IH27		26	100	100	2		M/F	MEDIUM BASE

IH28		26	100	100	2	l	M	Shrouded IDC

IH29		30	100	100	2		---	STAGGERED

IH30		30	100	100	2	l	---	M=IH31

IH31		30	100	100	2	l	---	M=IH30

IH32		40	100	100	2	l	---	M=IH33

IH33		40	100	100	2	l	---	M=IH32

IH34		50	100	100	2	l	---	M=IH36

IH35		50	100	100	2		---	STAGGERED

IH36		50	100	100	2	l	---	M=IH34

IH37		50	100	100	2		---

IH38		60	100	100	2	l	---	M=IH39

IH39		60	100	100	2	l	---	M=IH38

IH40		70	100	125	2	l	---	M=IH41	 STAGGERED

IH41		70	100	125	2	l	---	M=IH40 STAGGERED

IH42		70	100	100	2	l	---	M=IH43

IH43		70	100	100	2	l	---	M=IH42

IH44		70	100	100	2		---	STAGGERED

IH45		80	100	100	2	l	---	M=IH46

IH46		80	100	100	2	l	---	M=IH45

IH47		80	100	075	4	l	---	M=IH48

IH48	l	80	050	---	4	l	---	M=IH47 EDGE CONN

IH49		96	100	100	3	l	---	VME 1A/B/C...32A/B/C M=IH50

IH50		96	100	100	3	l	---	VME 1A/B/C...32A/B/C M=IH49

IHxx - HEADERS/SOCKETS

SYMB	SMC	PINS	C-C	R-R	ROW	R/A	M/F	DESCRIPTION / ALIAS           _

IH51		96	100	100	3	l	---	VME 1...96

IH52		96	100	100	3	l	---	VME 1A/B/C...32A/B/C

IH53	l	100	050	---	4	l	---	M=IH54 EDGE CONN

IH54		100	100	075	4	l	---	STAGGERED

IH55		122	100	125	3	l	---	M=IH56 STAGGERED

IH56		122	100	125	3	l	---	M=IH55 STAGGERED

IH57		180	100	100	3	l	---

IH58		180	100	100	3	l	---	SIM TO IH57

IH59		240	100	100	4	l	---

IH60		240	100	100	4	l	---

IH61		240	100	100	4	l	---	SIM TO IH57

IH62		2	156	---	1		---	LARGE BASE

IH63		4	156	---	1		---	LARGE BASE

IH64		9	156	---	1		---	LARGE BASE

IH65		24	156	---	1	l	---

IH66		12	200	---	1	l	---

IH67		180	100	100	3	l	---

IH68		50	100	100	2		---	SOCKET

IH69		8	100	100	2		---	SOCKET

IH70		10	100	100	2		---	SOCKET

IH71		14	100	100	2		---	SOCKET

IH72		14	100	100	2		---	HEADER

IH73	l	6	080	080	2		F	Samtec SSM-2MM 3 PIN

IH74	l	8	080	080	2		F	Samtec SSM-2MM

IH75	l	14	100	100	2	l	M	Samtec TSM-DH

IH76	l	6	080	080	2		F	Samtec SMM-2MM

IH77		28	049	049	2		M/F	Molex 1.25MM

IH78	l	2	100	100	2	l	M/F	Samtec SSM-102-X-SH

IH79		8	080	080	2		M	Samtec TMM-2MM

IH80		6	080	080	2		M	Samtec TMM-2MM

IH81		8	100	---	1		M/F	R-network, 8 pin

IH82	l	16	080	080	2		F	Samtec SSM-2MM

IH83		16	080	080	2		M	Samtec TMM-2MM

IH84		3	100	---	1		M/F

IH85		12	145	145	3		M/F	Molex	1625-12 PC/mount

IH86		14	049	---	1		M	Molex 53047-1410 1.25MM

IH87		34	100	100	2		M	Header

IH88		10	100	100	2		M	Header, 4-wall shroud

IH89		14	100	100	2		M	Header, 4-wall shroud

IH90		16	100	100	2		M	Header, 4-wall shroud

IH91		26	100	100	2		M	Header, 4-wall shroud

IH92		10	100	100	2	l	M	Sam MTSW-1-05-08-230-RA

IH93		10	100	100	2	l	M	IH92, 180° Body to pin rotate

IH94		10	049	---	1		M	Molex 53047-1010 1.25MM

IH95		14	050	---	1		F	Samtec socket strip

IH96		16	100	100	2		M/F

IH97		10	100	---	1		M/F

IH98		80	100	100	2		M/F

IH99		80	100	100	2		M/F	Split into 2 sections

IH100		4	100	100	2		M/F

SYMB	SMC	PINS	C-C	R-R	ROW	R/A	M/F	DESCRIPTION / ALIAS           _

IH101		14	100	100	2	l	M	Pin 1 upside down from norm

IH102		6	100	100	2		M/F

IH103		8	100	100	2		M/F

IH104		9	049	---	1	l	M	Molex 53048-0910

IH105		6	049	---	1	l	M	Molex 53048-0610

IH106		14	049	---	1	l	M	Molex 53048-1410

IH107		4						Spread header for factory PS

IH108	l	14	100	100	2	l	M	Board straddle header

IH109	l	10	19.67	116	2		M	Aromat AXK_F (P5KF)

IH110		24	110	100	3		M/F	Futaba RX connector

IH111		10	050	---	1		M/F	Ribbon interface

IH112	l	60	19.67	191	2		M/F	Amp 316112/917797 B-B conn

SRFxx - SPECIALIZED RF PARTS

SYMB	REF 		Freq		DESCRIPTION / ALIAS                                                _

SF1	13706		5.25-5.85G 	Wilkenson splitter/combiner

SF2	---		---		Wilkenson splitter/combiner

SF3	---		---		MSI filter

SF4	---		2.35G		Coupler

SF5	---		---		BPF

SF6	---		---		BPF

SF7	---		---		BPF

SF8	---		---		MSI outline, use SX4 or SX5

SF9	---		---		Multiplier outline, use SX3

SF10	---		---		BPF

SF11	---		---		Coupler

SF12	---		---		Coupler

SF13	---		---		Coupler

SF14	---		3.69-4.29G	BPF, C-Band

SF15	13624		4.4-5.0G	BPF, C-Band

SF16	13617		---		BPF, 1st LO

SF17	---		---		BPF, Long and thin

SF18	---		---		SF15 rotated 15°

SF19	13653		---		Filter, C-Band, fat poles up

SF20	13653		---		Filter, C-Band, fat poles down

SF21	13370/13679	---		Wire in a tube coupler

SF22	3040		---		Director/extender outline

SF23	13706		5.25-5.85G	BPF

SF24	13740/13727	---		BPF, doubler plug in board

SF25	13740/13727	---		SF24 converted to complement position

SF26	13613		4.4-5.0G	BPF with one bent leg

SF27	13727		---		Short and Cap BPF

SF28	13744/13743	---		2.2-2.4 Pass/4.4-7.2 Reject band stop filter